Materials Science Factory Regular Seminar

Micro-second force spectroscopy
 
MingdongDong*
 
 
Interdisciplinary Nanoscience Center (iNANO) and Department
of Physics and Astronomy, University of Aarhus, DK-8000 Aarhus C, Denmark
† To whom correspondence should be addressed. E-mail:  dong@inano.au.dk
 
 
Force spectroscopy is a powerful method to study the
structural and energetic dynamic of biomolecules. AFM-based force
spectroscopy
method allows measuring the interactions within individual molecules.
Recently
progress has been made in the measurement and characterization of dynamic
nanomechanical properties using specially designed torsional harmonic
cantilever (THC) sensor. The advantage of THC sensor based-AFM allows
running the
torsional mode to enhance harmonic signals in liquid environments. THC has
the
ability to recover the tip–sample force waveforms which provide high-speed
force–distance curves and allow specific material properties to be measured
with high spatial resolution. The entire period of the tip sample force
waveform is approx. 100 micro seconds and the interactions span 20 micro
seconds. Furthermore, the waveform exhibits low force noise of approx 10
pN. A
reduction in the peak interaction forces allows investigations of various
types
of proteins without causing them to be denatured. In addition, owing to the
microsecond duration of force loading, the mechanical properties derived
from
these waveforms will reflect molecular behaviour at the microsecond
timescale.
This talk will review the recently developed experimental technique and its
applications in quantitative imaging of membrane stiffness and the force
mapping based on molecular recognitions.